SPECIFICATION

Section 07900/079500

Previous 05800

Wabo®FastFloor

Model(s) “FJF”, “FJS”

Horizontal Expansion Control System

PART 1 - GENERAL

1.01	Work Included

A.	The work shall consist of furnishing and installing expansion joints in accordance with the details shown on the plans and the requirements of the specifications.  The joints are proprietary designs utilizing aluminum plate profiles.

B.	Related Work

- Cast-in-place concrete

- Miscellaneous and ornamental metals

- Flashing and sheet metal

1.02	Submittals

A.	Template Drawings - Submit typical joint cross-section(s) indicating pertinent dimensioning, general construction, component connections, and anchorage methods.

1.03	Product Delivery, Storage and Handling

A.	Deliver products in each manufacturer's original, intact, labeled containers and store under cover in a dry location until installed.  Store off the ground, protect from weather and construction activities.

1.04	Acceptable Manufacturer

- All joints shall be manufactured/supplied by Sika Emseal, 25 Bridle Lane, Westborough, MA 01581 USA.

B.	Alternate manufacturers and their products will be considered, provided they meet the design concept and are produced of materials that are equal to or superior to those called for in the base product specification.

C.	Any proposed alternate systems must be submitted and receive approval 21 days prior to the bid.  All post bid submittals will not be considered.  This submission shall be in accordance with MATERIALS AND SUBSTITUTIONS.

- Any manufacturer wishing to submit for prior approval must provide the following:

1.	A working 6” sample of the proposed system with a letter describing how system is considered superior to the specified system.

2.	A project proposal drawing that illustrates the recommended alternate system installed in the floor construction that is specific to the project.  Typical catalog cut sections will not be considered.

3.	Verifiable list of prior installations showing prior and successful experience with the proposed systems.

4.	Any substitution products not adhering to all specification requirements within, will not be considered.

  - Quality Assurance

- Warranty: Standard material limited warranty or project specific warranty available. Installation shall be in strict accordance with manufacturer’s technical specifications, details, installation instructions and general procedures in effect for normal intended usage and suitable applications under specific design movements and loading conditions.

- Manufacturer: Shall have a minimum ten (10) years experience specializing in the design and manufacture of Architectural Expansion Control Systems

- Application: Minimum 2 years [documented] experience in work of this Section

PART 2 - PRODUCT

2.01	General

A.	Provide floor joint cover consisting of metal profiles that utilize various metal finishes designed of width and thickness required to satisfy projects movement and loading requirements.  Secure cover plate to concrete floor slab by utilizing manufacturer’s recommended anchoring system.

Furnish Wabo® FastFloor, Model(s) “FJF”, “FJS” meeting ADA guidelines for interior joint locations as manufactured by Sika Emseal and as indicated on drawings.  Select Model based on requirements.

2.02	Components and Materials

A.	Aluminum Shapes - Material to conform to ASTM B209, alloy 6061-T6 (flush floor), or 5005-H34 (corner floor).

B.	Moisture Barrier (optional) - Shall be a fabric reinforced tear resistant clear vinyl sheet material.  Minimum thickness shall be .026”.

C.	Anchorage - Provide minimum ¼” diameter concrete expansion anchor at maximum 18” o.c. spacing to secure aluminum cover to floor slab.

D.	Accessories - Provide necessary and related parts, and fasteners required for complete installation.

E.	Fire Barrier Assembly - Designed to provide the required fire endurance rating, minimize passage of smoke and accommodate dynamic movement without stress or degradation to its components. Test system in maximum joint width incorporating a field splice in accordance with UL-2079. Supply Sika Emseal Emshield®  DFR2 (1-2-hour) , or DFR3 (3-hour) for joints ½” to 4” wide or Wabo®FireFlex (up to 20” wide and 2-hour fire rating) Fire Barrier Blanket System as governed by joint opening, test requirements and fire rating.

F.   Concrete Slab Repair (recommended) – Utilize a single component rapid strength repair mortar.

2.03	Fabrication

A.	All profiles to be shipped in standard 10 ft. lengths and shall be cut to length on jobsite where required. Profiles shall be miter cut in the field to conform to directional changes unless otherwise contracted with expansion joint manufacturer.

B.	Fire Barriers - Ship manufacturer’s standard assembly including fire caulks, sealants (if applicable) and hardware for the required hourly rating. Assemblies shall be miter cut in the field to accommodate changes in direction.

2.04	Finishes (Standard)

A.	The cover plate’s exposed surface shall receive a 60 grit brushed finish.  All other aluminum shapes shall be supplied in standard mill finish.

B.	Surfaces of aluminum profiles that will be in direct contact with concrete where moisture is present shall receive one coat of manufacturer’s recommended coating.

PART 3 - EXECUTION

3.01	Installation

A.	Protect all expansion joint component parts from damage during installation, placement of concrete and thereafter until completion of structure.

B.	Expansion joint systems shall be installed in strict accordance with the manufacturer's typical details and instructions along with the advice of their qualified representative.

C.	Expansion joint systems shall be set to the proper width for the ambient temperature at the time of installation.  This information is indicated in the contract plans.

3.02	Clean and Protect

A.	Protect system and its components during construction.  After work is complete in adjacent areas clean exposed surfaces with a suitable cleaner that will not harm or attack the finish.