SPECIFICATION

Section 07900

Wabo® SeismicSpan

# Models “APS”, “APX”

Seismic Expansion Control System

PART 1 - GENERAL

1.01	Work Included

A.	The work shall consist of furnishing and installing expansion joints in accordance with the details shown on the plans and the requirements of the specifications.  The joints are proprietary designs utilizing extruded elastomeric seals, base members and support plates.

B.	Related Work

- Cast-in-place concrete

- Miscellaneous and ornamental metals

- Flashing and sheet metal

- Sealants and caulking

1.02	Submittals

A.	Template Drawings - Submit typical seismic joint cross-section(s) indicating pertinent dimensioning, general construction, component connections, and anchorage methods.

1.03	Product Delivery, Storage and Handling

A.	Deliver products in manufacturer's original, intact, labeled containers and store under cover in a dry location until installed. Store off the ground, protect from weather and construction activities.

1.04	Acceptable Manufacturer

- All joints shall be as manufactured/supplied by Sika Emseal, 25 Bridle Lane, Westborough, MA 01581 USA

B.	Alternate manufacturers and their products will be considered, provided they meet the design concept and are produced of materials that are equal to or superior to those called for in the base product specification.

C.	Any proposed alternate systems must be submitted and receive approval 21 days prior to the bid. All post bid submittals will not be considered. This submission shall be in accordance with MATERIALS AND SUBSTITUTIONS.

Any manufacturer wishing to submit for prior approval must provide the following:

1.	A working 6” sample of the proposed system with a letter describing how system is considered superior to the specified system.

2.	A project proposal drawing that illustrates the recommended alternate system installed in the concrete deck that is specific to the project. Typical catalog cut sections will not be considered.

3.	Verifiable list of prior installations showing successful experience with the proposed systems.

4.	Any alternate products not adhering to all specification requirements within, will not be considered.

  - Quality Assurance

- Warranty: Standard material limited warranty or project specific warranty available. Installation shall be in strict accordance with manufacturer’s technical specifications, details, installation instructions and general procedures in effect for normal intended usage and suitable applications under specific design movements and loading conditions.

- Manufacturer: Shall have a minimum ten (10) years experience specializing in the design and manufacture of Architectural Expansion Control Systems

- Products: Expansion Control Systems must be installed with manufacturer’s blockout repair and infill material(s).

- Application: Minimum 2 years [documented] experience in work of this Section

PART 2 - PRODUCT

2.01	General

A.	Provide seismic control system incorporating elastomeric concrete capable of accommodating vehicular traffic and multi-directional seismic movement without stress to its components. Throughout normal service, the system shall resist the passage of moisture through the joint opening by utilizing manufacturer’s standard preformed elastomeric seal, that is mechanically locked into the aluminum base member cavity. A gutter system with drain tubes is optional and at the discretion of the engineer. The traffic bearing surface of the system shall consist of a flat cover plate with beveled edges designed of a width and thickness to satisfy projects movement and loading requirements. Secure cover plate to base member capable of HS-20 loading utilizing manufacturer’s pre-engineered heavy duty seismic-centering bar.

Furnish, Wabo® SeismicSpan Expansion Control System, Model “APS” or “APX” for deck and ramp areas designed for vehicular traffic as manufactured by Sika Emseal and as indicated on drawings.

2.02	Components and Materials

A.	Aluminum Extrusions - Material to conform to properties of ASTM B221, alloy 6005-T6 or 6063-T5.

B.	Aluminum Shapes - Material to conform to ASTM B209, alloy 6061-T6 or 5005-H34.

C.	Cover Plate - Provide minimum 3/8” thick plate with material to be aluminum conforming to ASTM B209, alloy 6061-T6. Slide plate to be secured to joint assembly utilizing a heavy duty self-centering bar that freely rotates in all directions. Preformed metal devices that utilize tension or compression to maintain and secure slide plate will not be allowed. Cover plate shall have beveled edges.

D.	Aluminum Flange Extension (Model “APX”) - Material shall be 3/16” minimum thickness conforming to ASTM B209, Alloy 5052-H32. Field attach to base member utilizing 1/4” diameter csk flathead machine screws spaced at 12” o.c. (maximum).

E.	Seismic-Centering Bar - Shall exhibit circular sphered ends that lock and slide inside the corresponding aluminum extrusion cavity to allow freedom of movement and flexure in all directions including vertical displacement.  Bar shall be molded or manufactured incorporating corrosion resistant nylon components with sphered ends and 1 1/2” wide cross member for standard applications. Spacing shall be a maximum of 18”o.c.

During seismic activity design centering bar to permit vertical displacement of metal cover during accelerated inward and outward movement without evidence of fatigue and permanent deformation. Concurrently provide secure connection between plate and underlying system components to maintain proper positioning and contact to adjoining surfaces.

Bar shall exhibit the following physical properties to demonstrate ability to resist corrosion and fatigue.

PHYSICAL PROPERTIES

Molded End Profile:

Material	:				Nylon

Color:						Black

Tensile Strength @ break:		ASTM D638   25,500 psi

Cross-Member:

Material:     				Pre-tempered spring steel

Damage Mitigation - Test Requirements:

Seismic-centering bar must exhibit ability to disengage (controlled release) from expansion joint edge member(s) when seismic movement exceeds the specified maximum allowable opening. Submit independent test report demonstrating required design of seismic-centering bar.

Requirements

a) Equipment: 	    	 	- Instron Machine

b) Orientation: 			- Specimen subjected to tensile load with cross member          parallel to direction of load

c) Specimens:        	          	- Test 4(min)– select at random

d) Disengagement range (lbs) :	- 800 (min.) – 1250 (max.)

- Moisture Barrier - Extruded elastomeric profile shall be designed utilizing a serpentine configuration allowing maximum movement and flexibility. Its side lugs shall mechanically lock into a corresponding aluminum profile. Material shall be a quality flame retardant vinyl with a minimum material thickness of .063”. Install profile using Wabo®PrimaLub Adhesive.

- Fire Barrier Assembly  - Designed to provide the required fire endurance rating, minimize passage of smoke and accommodate dynamic movement without stress or degradation to its components. Test system in maximum joint width incorporating a field splice in accordance with UL-2079. Supply Emshield® DFR Fire Barrier as governed by joint opening and fire rating up to 3-hour UL listing and ½” – 6” wide nominal joint opening. Provide Wabo®FireFlex Barrier as governed by joint opening and fire rating up to 2-hour fire rating and 20” wide nominal joint opening.

G.	Vibration Reduction Gasket (Model “APS” only) - Provide manufacturers extruded profile designed to lock into base member’s continuous serrated channel. Material shall be EPDM Shore Hardings 80A.

- Anchorage - Provide minimum 3/8” diameter concrete expansion anchor at maximum 18” o.c. spacing to secure aluminum base member to concrete slab.

I.   Blockout Repair - Utilize a single component rapid strength repair mortar.

J. Blockout Infill - Utilize WaboCrete Parking Series elastomeric concrete.

- Elastomeric Concrete:	(WaboCrete Parking Series)

Material shall be an ambient cure, 100% solids, two component polyurethane with pregraded aggregate mix exhibiting the physical properties listed in the tables below.  When properly mixed and poured, the elastomeric concrete cures rapidly, flows and fills any voids, spalls or irregularities forming a monolithic unit.

Base Color: Gray

1.  Elastomeric cured binder and aggregate shall meet the following physical properties:

PHYSICAL PROPERTIES      ASTM METHOD       REQUIREMENTS          TEST RESULTS

Compressive Strength             D695 Mod                    2200                             2723 psi

Resilience at 5% deflection      D695                             90 (min.)                         96.7%

Adhesion to concrete:              C190

Dry Bond                                                  400                              451.3 psi

Wet Bond                                                  250                             353.9 psi

Impact Resistance                  Steel ball drop (.375” th. Disc / dry steel plate)

At  -20 deg F(-29C)                                         no cracks at 5 ft                7.0 ft-lbs

At   32 deg F (0C)                                            no cracks at 5 ft              10.0 ft-lbs

At 158 deg F (70C)                                          no cracks at 5 ft              10.0 ft-lbs

L.	Bonding Agent

Provide manufacturer’s two component, 100 percent solids bonding agent and apply to the sides and base of the preformed concrete blockouts prior to placement of WaboCrete elastomeric concrete header. Utilize on all exterior applications. For interior applications utilize where moisture is present in the concrete. Store, mix and apply in accordance with manufacture’s system data sheet.

Liquid components shall be identified by the following information:

Part A - Resin; Color:  Clear

Part B - Activator; Color:  Tan

M.  Accessories - Provide necessary sealants and assembly hardware required for complete installation.

2.03	Fabrication

A.	Aluminum extrusions and generic profiles to be shipped in standard 10 ft. lengths and shall be cut to length on jobsite where required. All profiles shall be miter cut in the field to conform to directional changes unless otherwise contracted with expansion joint manufacturer.

B.	Elastomeric moisture barrier shall be shipped in the longest practical continuous length in manufacturer’s standard shipping carton.

C.  Fire Barriers (where applicable) - Ship manufacturer’s standard assembly including fire caulks, sealants (if applicable) and hardware for the required hourly rating. Assemblies shall be miter cut in the field to accommodate changes in direction.

D.  Elastomeric Concrete Blockout Infill - Activator packaged in one-half gallon containers (Part A), resin packaged in gallon containers (Part B), 60 pound containers of aggregate (Part C) and broadcast aggregate shipped on wooden pallets, shrink wrapped.

2.04	Finishes

- The cover plate’s exposed surface shall receive a 60 grit brushed finish. All other aluminum extrusions and shapes shall be supplied in standard mill finish.

- Surfaces of aluminum extrusions that will be in direct contact with concrete where moisture is present shall receive one coat of manufacturer’s recommended coating.

- Elastomeric Concrete Blockout Infill :

-  Elastomeric Concrete base infill material shall be supplied in standard color: Gray.

- Elastomeric Concrete base infill material after field application of broadcast aggregate granules:  Gray-textured.

PART 3 - EXECUTION

3.01	Installation

A.	Protect all expansion joint component parts from damage during installation, placing of concrete and thereafter until completion of structure.

B.	Expansion joint systems shall be installed in strict accordance with the manufacturer's typical details and instructions along with the advice of their qualified representative.

C.  Expansion joint systems shall be set to the proper width for the ambient temperature at the time of installation. This information is indicated in the contract plans.

D.   Install all Expansion Control Systems utilizing manufacturer’s blockout repair and infill material(s).

- Expansion joint surface areas two feet on each side of joint gap shall be finish graded perpendicular to joint gap creating flush slab-to-slab transition. Elevations on each side shall be identical.

3.02	Clean and Protect

A.	Protect system and its components during construction. After work is complete, clean exposed surfaces with a suitable cleaner that will not harm or attack the finish.